Week 4 Submission Assignment

Qibang Liu

Trine University

IS 5203

Professor Khodor Dakrouni

February 1, 2026

In modern networks IEEE 802.1Q VLAN tagging is very important since many logical networks use the same physical infrastructure but are logically isolated. Today's enterprise networks are rarely housed in a single switch. Several switches must be interconnected and simultaneously handle traffic for many VLANs. It is solved by IEEE 802.1Q which tags Ethernet frames as they travel over a trunk link so switches know to forward the frame to the correct VLAN. Without VLAN tagging traffic from different VLANs would mix together on inter-switch links breaking segmentation & possibly breaking performance/security.

An example is the trunk link between access switches and distribution switch in an office building. Access ports go to end devices and send untagged frames whereas trunk ports between switches tag frames for staff, guests and management VLANs using IEEE 802.1Q. Another example is router-on-a-stick, using VLAN tags to route traffic between multiple VLANs with one router interface set up as an 802.1Q trunk.